TRACK C

Geometry / Scale Stabilization

Output: Reduced auto-calibration extremes and jump spikes on regression videos

# 1. Objective

The 2D-to-3D coordinate transform is the single biggest source of physics validation failures. Auto-calibration correction factors reach 80–243x (should be close to 1.0x). This means the pixel-to-meter scaling is off by two orders of magnitude. Fix the geometry pipeline so that raw 3D trajectories produce speeds in the physically plausible range (15–70 km/h for slalom) without needing massive auto-calibration corrections.

# 2. Current Failure Modes

Measured on the 3 regression videos (2907, 2909, 2911) with stabilize=True:

| Problem | Symptom | Current Value | Expected |
| --- | --- | --- | --- |
| Scale estimation | Auto-cal correction | 6x – 243x | 0.8x – 1.5x |
| Speed spikes | Max speed | 419 – 3123 km/h | < 70 km/h (SL) |
| G-force spikes | Max G-force | 355 – 1479 G | < 5 G |
| Position jumps | Max jump (m) | 4.78 – 28.94 | < 1.0 m/frame |

# 3. What Needs to Be Solved

## 3.1 Gate Spacing Assumption Audit

The pipeline assumes gate_spacing_m=12.0 (distance between consecutive slalom gates). This is the single most impactful parameter for scale.

- Verify the 12m assumption. FIS slalom regulations specify 0.75m–13m between gates, with typical spacing around 9–12m depending on terrain. If the test videos use a training course (not FIS), the spacing could be different. Measure actual gate spacing if possible, or parameterize as a range.

- Test sensitivity. Run the pipeline on one regression video with gate_spacing_m = 8, 10, 12, 14. Record the auto-cal correction factor and P90 speed for each. This tells you how sensitive the output is to this parameter.

- Implement adaptive spacing. If the model detects N gates, compute the median pixel distance between consecutive gate pairs. Use this ratio rather than a fixed assumption. This is more robust than assuming a single fixed spacing.

## 3.2 Camera Pitch Correction

Currently hardcoded at 6 degrees (camera_pitch_deg=6). Wrong pitch angle directly scales the Y-axis transform.

- Understand the effect. Camera pitch correction multiplies the Y-scale by cos(pitch). At 6 degrees, this is cos(6) = 0.9945 (negligible). At 30 degrees, cos(30) = 0.866 (13% correction). If the actual camera angle is steep (broadcast cameras on tripods overlooking the course), 6 degrees may be far too low.

- Estimate from gate geometry. If gates are visible at both top and bottom of frame, the apparent foreshortening ratio gives an estimate of camera pitch. Implement a function that computes pitch from the ratio of gate sizes at top vs bottom of frame.

- Test with pitch = 0, 10, 20, 30 degrees on each regression video. Record the auto-cal correction factor. Find the pitch that minimizes the correction factor.

## 3.3 Homography vs Scale Projection

The pipeline currently defaults to projection="scale" (linear Y-mapping). The alternative is full 4-point homography which is more accurate in theory but more fragile.

- Compare both modes on all 3 regression videos. Record auto-cal correction, max speed, max G-force for each mode. Pick the mode that consistently produces lower correction factors.

- If homography is better, add sanity checks. Before applying a computed homography, verify that (a) the determinant is positive (no flipping), (b) the condition number is < 1000 (not ill-conditioned), (c) transformed gate positions map to plausible real-world coordinates.

## 3.4 Dynamic Scale (Phase 4) Tuning

DynamicScaleTransform recalculates pixels-to-meters per frame. This should help as perspective changes while the skier moves down the slope, but it can also amplify noise if gate detections jitter.

- Add scale smoothing. Apply a median filter (window=15 frames) to the per-frame scale values before applying them. This prevents single-frame gate jitter from creating scale spikes.

- Add scale bounds. Reject any per-frame scale that is more than 2x or less than 0.5x the median scale. Replace rejected frames with interpolated values.

- Log scale time series. For each processed video, save the per-frame scale values to the output JSON so they can be plotted and inspected for anomalies.

## 3.5 Auto-Calibration Guard Rails

The current auto-calibration applies a correction factor based on P90 speed. This is a safety net, not a fix. If the correction factor is > 3x, something upstream is fundamentally wrong.

- Add a warning when correction > 3x. Print a diagnostic message listing the likely causes: wrong gate spacing, wrong camera pitch, too few gates detected, gates detected on wrong objects.

- Cap the correction factor at 5x. If the correction would be > 5x, refuse to apply it and instead flag the video as “unable to calibrate.” This prevents the auto-calibration from masking a fundamentally broken upstream.

# 4. How to Check Output

## 4.1 Primary Success Metric

Run the 3 regression videos through the pipeline with stabilize=True. Record auto-calibration correction factors.

| Metric | Target | Current |
| --- | --- | --- |
| Auto-cal correction (mean) | < 5x | 84.8x |
| Auto-cal correction (max) | < 10x | 243x |
| P90 speed before auto-cal | 15–70 km/h | ~22 km/h (after cal) |
| Max speed after auto-cal | < 100 km/h | 419–3123 km/h |

## 4.2 Deliverables

- Sensitivity analysis: docs/reports/geometry_sensitivity_YYYYMMDD.md showing gate_spacing, camera_pitch, and projection mode sweeps

- Updated transform.py with scale smoothing, scale bounds, and improved pitch estimation

- Updated pipeline.py with auto-cal guard rails (cap at 5x, diagnostic warnings)

- Per-frame scale time series added to output JSON

# 5. Files You Will Touch

- ski_racing/transform.py — main work area (HomographyTransform, DynamicScaleTransform)

- ski_racing/pipeline.py — auto-calibration section (_auto_calibrate_scale method)

- scripts/process_video.py — add new CLI flags if needed (--gate-spacing, --camera-pitch)

# 6. Dependencies

- Track B: Better gate detection directly improves geometry estimation. But do not wait for Track B — fix the geometry pipeline with current detections first.

- Track D: Smoother trajectories reduce apparent speed/G-force spikes, which will make it easier to tell if geometry fixes are working. Coordinate on which track’s improvements to merge first.